A simulator for the Elliott 903 and its Algol system.

21-MAY-1996.

Terry Froggatt, 12 Leawood Road, Fleet, Hants, GU13 8AL, UK.

Tel. +44 (0)1252 613996

Don Hunter, The Old Vicarage, Elmdon, Saffron Walden, Essex CB11 4LT.

Tel. +44 (0)1763 838472 (don@gnhunter.demon.co.uk)

The Elliott 903 made its appearance in about 1965 and was an 18-bit

word machine with up to 64K words of store, although the basic model

had only 8K. It usually came with an on-line Teletype, paper tape

reader and punch, and perhaps a plotter.

The store cycle time was six microseconds and, as an economy measure,

both the sequence control and index registers were held in the store

rather than provided in the hardware. This led to a typical instruction

time of 25 to 30 microseconds, but there was a bonus when using the

machine for real-time work since at least these two registers did not

need to be preserved between the four interrupt levels, as they were

already in four separate pairs of storage locations.

A later machine, the 905, did have these registers in hardware and a

two microsecond store cycle time, and could address 128K words of

store.

Languages available included Algol, Basic, Coral, Fortran and SIR

(Symbolic Input Routine, the assembler). Probably 1000 machines were

sold, especially the miniature version which was installed in military

planes, and in tanks and ships. The 903 was also used to control

laboratory equipment and in schools.

Terry Froggatt first wrote an Ada simulator for the Elliott 900 series

in 1985, partly to evaluate early compilers and partly as an exercise

in writing portable code. This 1992 version was modified for Meridian

Ada and copes with simulating the full 64K words of store. It is run

from the DOS prompt.

It did not have any software to run with it, although a subset of SIR

was included without having to be held in the simulated store. The main

missing SIR features are label names and the constants pool.

Here is a modification which loads the Algol system, together with a

few demonstration programs. What kind of Algol is it ? Well it comes

with the Elliott I/O system, 18-bit integers and reals with 8

significant decimal places. It was written by CAP and Elliotts in

1966/67 as an IFIP subset Algol for an 8K machine and was derived

loosely from KDF9 Whetstone Algol. Norman Spink at Elliotts

subsequently turned it into a Load-and-Go system for a 16K machine and

I removed most of the IFIP restrictions with patches in the spare

space.

Translation speed was 100 characters per second and programs ran at 60

statements per second on the Whetstone benchmark, one third as fast as

its KDF9 parent. On a 486 running at 100MHz the simulator runs at about

half the speed of an Elliott 903, when the Ada compilation includes the

usual checks. Simulator times below refer to this case.

The 903 outputs one character at a time but the simulator outputs one

line at a time. Be patient occasionally. When translating a program

there is no output until translation has finished.

Its main restrictions are given in appendix A.

Installation

------------

To install it just copy everything on the disk into a directory of your

choice, for example ELLIOTT. To run a program its text must be in the

file called EIN.DAT (Elliott INput), so copy it into there from one of

the demonstration programs and then type SIM900AL. See the example in

appendix D.

The files M0.DAT and M1.DAT hold the SIR code for modules 0 and 1, with

8K words each.

The simulator can be used for SIR as well as for Algol. TEST.DAT is

some SIR code for displaying 64-word checksums of either module 0 or 1

(see its comments). However, the simulator code keeps looking out for

two stopping addresses, 1727 and 7943, so as to take special action for

Algol, but this is unlikely to be a nuisance.

Character set

-------------

The character set includes upper and lower case letters, but they are

regarded as the same. On output all letters are in upper case. The

characters #  $ % & \ are legal in strings (# and  both come out as

#). The character back-arrow, represented by _ in ASCII is used to

cancel an input line of text, by placing it as the last character in

the line. This was arranged to allow corrections of programs typed on a

Teletype.

The representation of Algol symbols is as shown and this allows spaces

to occur in names (the stropping convention is double quotes around

keywords).

begin  end        "BEGIN"  "END"  or "begin" "end"

<=                "LE"

>=                "GE"

/=                "NE"

and               "AND"

or                "OR"

not               "NOT"

equiv             "EQUIV"

impl              "IMPL"

string quotes     ' and @

subscript ten     ?

*+-/()[]<=>:;.,   stand for themselves

exponentiation    ^

div               "DIV"

The six-bit internal character code is given in Appendix E.

Library procedures

------------------

The standard library contains ARCTAN, COS, INSTRING, LOWBOUND,

OUTSTRING, RANGE, SIN, SQRT. There is no external library as there was

in the real machine, but there could be (see DEMO8).

The built-in procedures are ABS, ALIGNED, DIGITS, ENTIER, EXP,

FREEPOINT, LN, PREFIX, PUNCH, READER, SAME LINE, SCALED, SIGN, STOP,

WAIT.

Some names are known to the translator but are not in the library.

These are ADVANCE, BUFFER, CENCHARACTER, DECODE, DRAW LINE, MOVE PEN,

PAGE, SET ORIGIN and WAY.

Demonstration programs

----------------------

The programs have some initial directives for the benefit of the

simulator. If you write a program you must adopt the directives from

whatever example seems appropriate. There are ten demonstration

programs called DEMO1.DAT to DEMO10.DAT.

DEMO1 is just a table of squares.

DEMO2 is Knuth's Man or Boy test, running up to k=11. It takes about

seven or eight minutes and uses about 40K or 50K words of store. The

crux is at k=4. For each successive value of k the recursion depth and

evaluation time double. The recursion depth reaches about 1024 in both

the A and B real procedures. Notice the patch at the front of DEMO2 to

inform the Algol system that 64K of store is available.

DEMO3 is Knuth and Merner's program which prints several primes in one

statement. It has been arranged to print 10 primes to a line and ten

primes in all, but could easily be edited to do more. If so, the first

line takes a couple of minutes and a second line about 20 or 25 minutes

on a 903. Remember that output occurs only when a complete line is

ready. To see progress you can declare another integer, say t, set it

to zero at the start, and increment and print it inside the integer

procedure G. In this way you retain control to return to DOS with a

control C even if you have not installed breakout.

DEMO4 is taken from the article by Bryan Higman but adapted to evaluate

PI by integrating one eighth of the volume of a sphere. In this example

the fact that local variables are in static storage is evaded by

holding them in the procedure called LOCAL. There is a lot of use of

parameter comments where this construct:

)a comment :(

boils down to a comma.

DEMO5 illustrates the use of lower case characters and reading input

data.

DEMO6 shows special characters in strings.

DEMO7 shows library operations (see description under DEMO8). Note that

REVERT is declared but not called and therefore not loaded; FLOOR is

declared and called in the outermost block and therefore it is loaded;

ADVANCE, BUFFER and DECODE do not have to be declared, they are called

and so they are loaded.

DEMO8 is adapted from the Pascal program to calculate the rising and

setting times of the Sun and Moon given in the book, Astronomy on the

Personal Computer, by Montenbruck and Pfleger. It takes nearly two

minutes to print each output line on a 903 and probably 3 to 4 minutes

on the simulator. It illustrates several features:

1. An initial patch to the translator to make the code compact, but to

disallow call-by-name. The default is to have "5 86" instead of

"8 86" at location 8273.

2. A directive, ADRLB, which prepares for input of relocatable binary

code after the Algol text, in addition to the standard library

stuff.

3. The declarations "CODE" ... "ALGOL" which declare the headings for

assembly-coded procedures supplied as relocatable binary code.

4. The relocatable binary code itself. Each integer represents one row

of tape. About ten blank rows (zeros) precede and follow each

section.

5. The loader halt code is three integers to make up one 903 word.

Special things happen if the word is non-zero (if less than 8 then

set the word into the loader options, or else treat the number as an

address and start at that address).

6. It gives an idea of the maximum program size. Compare the address

given for NEXT with the bottom of the Name List which starts at

location 7512, but remember that this grows downwards during

translation except when names are discarded at the end of a block.

Each name needs four words.

DEMO9 is Quicksort, unchanged but for the declaration of local

variables and even the layout of the original has been more or less

followed. It just sorts an array of ten real numbers into order, but

the data tape could be changed to alter the array size. It uses the

Wichmann-Hill random number generator.

DEMO10 calculates the function mentioned in Knuth's Art of Computer

Programming Vol. 2 Ed. 2, for estimating the size of a factor base in

either the Continued Fraction or Multiple Polynomial Quadratic Sieve

factorisation programs. Be careful when running it with an increased

number of steps as the run time goes up as the square of this.

Starting addresses and initial directives

-----------------------------------------

In the real machine one can start at several different addresses in

succession, but in the simulator the run must be done in one go as

shown here. When particular stopping addresses are reached the

simulator launches the next step.

8  Translate and then run, with the standard library.

12  Translate and then run, without the standard library (more space is

available).

16  Translate in "Report Mode" as a syntax check, and also so as to

relate run-time error reports to the place in the code where

they occur.

ADRLB  Prepares the simulator to input RLB code after the end of the

Algol text (see DEMO7, DEMO8).

KEYB   Returns control to the keyboard.

EIN    Returns control to the text input file.

QUIT   Returns to DOS.

Starting addresses on the real 903

----------------------------------

14  Add a piece of relocatable code (done automatically when ADRLB

is given).

17  Translate to "paper tape", but must have OPBIN instead of OPASC

at the front of the file. There is not much point in translating

to paper tape, although one could generate separately compiled

procedures this way (beyond the scope of this note).

18  Re-establish the library (expects RLB tape input).

19  Checksum the store and alter location 8 to check the checksum

before translation begins (session to session preservation).

25  Freeze the translator known names (ie can add to them).

Miscellaneous

-------------

Integers are initialised to 78643 and reals to 1.351087?15 so as to

catch some uses before assignment. The largest real is about 9?18.

When reading a number from the input the character "!" causes all

following characters up to  a row of blank tape to be transferred

from input to output. This acts as a title for different data cases.

For the simulator I do not know how to represent blank tape as ASCII

characters.

Stick to input device 1 (Reader) rather than 3 (Teletype), because

DECODE only works for the Reader. Output devices 4 and 5 are the line

printer and plotter which are not available on the simulator.

Relocatable tapes are checksummed, but the check is only applied when

adding a missing procedure by a start at 14, NOT when translating by

a start at 12, on the grounds that the library material must be all

right anyway I suppose (although it might have been mis-read).

As mentioned above (see (1) under DEMO8), if Call by Name is not needed

then the code can be made shorter and it runs slightly faster with the

initial patch

^8273

8  86

The Algol system must be told that more store is available in order to

use it. Place this patch

^27

+65522

near the front of the program (see DEMO2). This just extends the data

space, for example one can declare an integer array with 60000

elements. It would be possible to rearrange the store layout, allowing

code to occupy a whole store module, say module 2, instead of just

part of module 0. This was done with a different Algol version and more

than doubles the maximum program size, at some cost in addressing

complexity.

APPENDIX A    Restrictions

1.  The names ADRLB, EIN and KEYB must not be used even in comments,

because they get intercepted by the simulator.

2.  Local variables are in static storage, but see DEMO4 for a way

of getting around this (you just have a local procedure).

3.  Everything must be declared before it is used.

4.  Switch lists may only contain labels, not designational

expressions.

5.  Unsigned integers may not be used as labels.

6.  The controlled variable in a for clause may not be subscripted.

7.  Own is not allowed.

8.  All formal parameters of a procedure must be specified.

9.  A call of a type procedure can only occur in an expression.

10. Only the first six characters are significant in an identifier.

11. The identifiers checkr, checki, checkb and checks are reserved.

12. Only 14 parameters are allowed in a procedure call.

13. The translator needs help with calls of FORMAL procedures; its

call must indicate which parameters are called by name and which

by value. It assumes a call by value is meant when a closing

bracket, round or square, precedes the comma or closing round

bracket. This means that an extra bracket must sometimes be used.

14. "IF" A+B "THEN" is indistinguishable from "IF" A+B > 0 "THEN".

APPENDIX B     Errors at translation time

When an error is reported, the error number, the line number and the

line of text are given, together with an upward pointing arrow to

show where the error occurred. Translation continues and further

errors may be given. An example follows:

ERROR NO    18

LINE NO      7

W := 23;

^

meaning that the variable W has not been declared. There was so little

store in the original machine that the system had to be generous with

error information.

The line number starts at 1 for the title line and includes only

lines with printable stuff on them.

Some errors are reported from the loader and consist of only two

characters, eg FE means program is too large. These errors are given

at the end of this appendix.

1.   "READ" misplaced

2.   "PRINT" misplaced

3.   Constant or expression in "READ" list.

4.   Wrong delimiter in "SWITCH" declaration.

5.   Illegal actual parameter.

6.   Too many parameters to a procedure.

7.   Illegal number.

8.   Integer constant too big, but I think it is converted to real

now.

9.   Two statements in the same block are prefixed by the same label.

10.  Identifier or constant not as expected.

11.  Letter, digit or subscript ten misused.

12.  "TRUE" or "FALSE" follows an identifier or constant.

13.  "COMMENT" does not follow semicolon or "BEGIN".

14.  An internal error.

15.  Unrecognised basic symbol, eg caused by "FRED".

16.  No assignment within a type procedure to procedure name.

17.  Identifier in value or spec. part is not a formal parameter.

18.  Use of undeclared identifier.

19.  Illegal symbol.

20.  Non procedure identifier used as a statement.

21.  ":=" omitted from for clause.

22.  Illegal use of label name.

23.  Inadmissible array declaration.

24.  <switch name> not an actual parameter nor preceded by goto.

25.  Non type procedure as function designator.

26.  "SWITCH" misplaced.

27.  Declaration without identifier.

28.  ":=" preceded by a constant or used inside an expression.

29.  ":" in type or switch declaration misused.

30.  Adjacent delimiters inadmissible.

31.  Constant before ":=" or "[", or constant or string name in read

list.

32.  Item other than a non-type procedure used as a statement.

33.  Identifier or constant follows a closing round or square

bracket.

34.  Relation on each side of a simple arithmetic expression.

35.  Illegal statement, delimiter misused.

36.  Declaration starts incorrectly.

37.  Error between "FOR" and ":=".

38.  Missing array or switch name, or "[" misplaced.

39.  ";" misused in array declaration.

40.  "END" misused.

41.  Local identifier used in array bounds.

42.  "GOTO" follows an identifier or constant.

43.  Wrong "FOR" clause preceding "DO".

44.  "FOR" misused.

45.  Misused boolean constant.

46.  Assignment to procedure identifier outside procedure body.

47.  "REAL", "INTEGER" or "BOOLEAN" misplaced.

48.  Identifier declared twice in same block head.

49.  Blank parameter.

50.  No "BEGIN" at start of program.

51.  Wrong number of subscripts or parameters.

52.  ":=" appears in actual parameter list.

53.  Statement ends incorrectly.

54.  Declaration follows statement.

55.  ":", "GOTO" or "FOR" used in expression.

56.  Illegal parameter comment or ")" precedes identifier.

57.  Wrong use of delimiter.

58.  Relational or logical operator used as an arithmetic operator.

59.  Illegal use of logical operator.

60.  Omission or error precedes "BEGIN", or "BEGIN" follows ":=".

61.  "(" misplaced or missing procedure name.

62.  Function designator as designational expression.

63.  Misplaced declarator.

64.  Subscripted variable as statement.

65.  Illegal specifier.

66.  Misused comma or colon in an expression.

67.  "IF" misused or used in "READ" or "PRINT" list.

68.  "IF" used in type declaration.

69.  Corresponding "IF" has been omitted, or conditional expression

without an "ELSE".

70.  Corresponding "THEN" missing.

71.  Illegal character in inner string, or missing close quotes in

previous string.

72.  "ARRAY" misplaced.

73.  "[" not preceded by an identifier.

74.  Unmatched "]".

75.  Upper bound missing in an array declaration.

76.  Illegal type declaration.

77.  Illegal array list.

78.  Corresponding "FOR" missing.

79.  A jump is made to a label declared, but not placed in the block

that ends here.

80.  "STEP", "UNTIL" or "WHILE" misused in a "FOR" statement.

81.  Misused ")" other than in an expression.

82.  ")" misplaced or unmatched

83.  Program too complex, ie some statemnet is too complicated.

84.  Wrong delimiter after procedure statement.

85.  Program too large, ie it contains too many names, labels,

constants or switches.

86.  Error before "PROCEDURE".

87.  Repeated formal parameter.

88.  Wrong formal parameter delimiter.

89.  In a call of a formal procedure, one of its parameters is LN or

EXP.

90.  Wrong delimiter in value or specification part.

91.  Input buffer overflow, ie more than 120 characters in a line.

92.  Formal parameter has not appeared in the specification part.

93.  Declaration terminated by "END" or containing "BEGIN".

94.  A formal parameter which is a switch, string or procedure is

called by value.

95.  Switch designator has more than one subscript.

96.  Wrong "FOR" clause.

97.  "THEN" misused.

98.  Illegal character or parity error. The character is replaced by

the back-arrow character in the displayed line but "^" is not

printed beneath it. In the simulator the back-arrow character is

replaced by underscore "_".

99.  Current use of identifier inconsistent with previous uses.

100. Conditional expression needs parentheses.

101. Wrong delimiter after procedure identifier in procedure

declaration.

102. No ";" between formal parameter part and value or specification

part.

103. Commas or colons wrong in array bounds.

104. "DIV" used with a real argument.

105. Illegal parameter delimiter after a string.

106. Integer lables not allowed.

107. <reserved>

108. <reserved>

109. Constant should not be used in procedure heading.

110. Wrong specification part.

111. Different number of parameters from previous use of formal

procedure or wrong number of subcripts.

112. Mixed type in multiple assignment.

Load time errors

FA   Misread or mispunched paper tape.

FC   Two procedures (at least one of which is a library procedure) have

the same name.

FD   As FA.

FE   Program too large to load.

FF   As FA.

FG   Call-by-name when in "optimise mode". You have to patch the

translator to get it to run in this mode where it no longer

outputs NOP words at each parameter in a procedure call. See

DEMO8.

FU   Missing library procedure.

APPENDIX C     Errors at run time

A run time error looks like this:

ERROR NO  ADR   RET

9    48      0

where ADR represents the place where the error occurred and RET is the

place where the call to the current piece of code was. These numbers

refer you to the output of running in Report Mode (start at 16) when

you get this for DEMO9 if you edit it to have an initial JUMP 16

instead of JUMP 8.

SIM900AL:  Version of 18-MAY-1996

SIM900AL:  Loading module 0

SIM900AL:  Loading module 1

SIM900AL:  Simulation Starting

QUICKS

P EXCHAN    ADR   70

E           ADR   82

P WICHMA    ADR   84

E           ADR  187

P RANDOM    ADR  189

E           ADR  237

P PARTIT    ADR  239

L UP        ADR  265

L DOWN      ADR  288

L CHANGE    ADR  312

E           ADR  348

E           ADR  379

E           ADR  410

E           ADR  410

P QUICKS    ADR  412

P LOCAL     ADR  417

E           ADR  452

E           ADR  452

E           ADR  458

E           ADR  555

E           ADR  556

PROGRAM  585

SCALARS   17

Stop at        +14008

SIM900AL:  Character Ignored

SIM900AL:  Simulation Completed

where a program called Quick Sort has various procedures indicated

above by being preceded by the letter P. Although the program name does

not conflict with the procedure name, it could conflict with the name

of a library procedure. Labels are preceded by the letter L and "END"s

are given by the Es, together with the overall length and the space

occupied by all the scalars.

1.   Parameter mismatch

2.   Space overflow, too much claim on store.

3.   Integer overflow.

4.   Jump error, ie switch subscript outside range.

5.   Subscript error.

6.   Illegal symbol inside inner string quotes.

7.   Attempt to output non-standard floating point number.

8.   Illegal character or ' found when reading a number.

9.   Real overflow.

10.  Invalid argument to sin or cos.

11.  Negative argument to sqrt.

12.  Argument > 40 for exp.

13.  Negative argument for ln.

14.  Illegal character on data tape.

15.  Parity error on data tape.

16.  Input buffer overflow, ie longer than 120 characters.

17.  Numeric character encountered before ' on instring.

18.  Illegal form of number, eg two decimal points.

19.  A^B with A and B real and A<0.

20.  Corrupt program, possibly because of assembly coded bit.

21.  Attempt to assign to formal parameter which is a constant.

22.  Range of array subscript bounds is negative.

23.  Instring or outstring error.

24.  Attempt to jump to label in inner block.

25.  (Translator version incompatible with run-time system.)

APPENDIX D     Example run.

C:\ELLIOTT>copy demo5.dat ein.dat

C:\ELLIOTT>sim900al

SIM900AL:  Version of 18-MAY-1996

SIM900AL:  Loading module 0

SIM900AL:  Loading module 1

SIM900AL:  Simulation starting

SINETA  4540

QACODL  4614

QAVNDA  4623

FIRST  NEXT

4540   4627

SIM900AL:  Machine Reset

SINETA

X     SIN(X)

10   0.173648

20   0.342020

30   0.500000

40   0.642788

50   0.766044

60   0.866025

70   0.939693

80   0.984888

90   1.000000

FINISH

Stop at        +1727

SIM900AL:  Read Beyond File

SIM900AL:  Simulation Completed

APPENDIX E     Internal character code.

In this appendix the description of a character as it appears on an

Elliott Teletype is sometimes given above its ASCII equivalent where

they differ. The rows and columns are laid out in octal.

00     10     20     30     40     50     60     70

0   space    (      0      8   cls qte   H      P      X

@

1   nlcr     )      1      9      A      I      Q      Y

2     "      *      2      :      B      J      R      Z

3   half     +      3      ;      C      K      S      [

(not sure)

4     $      ,      4      <      D      L      T

# or

5     %      -      5      =      E      M      U      ]

6     &      .      6      >      F      N      V      ^

7     '      /      7  sub ten    G      O      W      ?

?                       (this is odd)

APPENDIX F     903 order code summary.

The most significant bit is the modifier bit, the next four are

function bits and the lowest 13 are address bits. A is the accumulator

and Q is the right half of the double length (A, Q) register. The

sequence control register is S.

/          B-modification     In the first stage of B-modification the

Q register is affected in an undefined

manner, consequently the effect of a

modified 3 instruction is undefined.

0          Set B register     B := Q := m

and Q register

1          Add to A           A := A + m

2          Negate and add     A := m - A

Q := m

3          Store Q            m := Q div 2 (Q always treated as

positive)

4          Load A             A := m

5          Store A            m := A

6          Collate with A     A := A & m

7          Jump if A = 0      S := m    if A = 0

8          Jump               S := m

9          Jump if A < 0      S := m    if A < 0

10          Count in store     m := m + 1

11          Store S            m := S

12          Multiply           (A, Q) := A * m

13          Divide             A := (A, Q) / m rounded

14          n                  Shift operations

0 < n < 49         Shift (A, Q) left  n places

8164 < n < 8192    Shift (A, Q) right (8192 - n) places

Other 14 instructions exist for block

transfers to magnetic tape and

plotters

15          z                  IO operations

z = 2048           Shift A left 7 places; OR in character

from paper tape reader to lowest 8 bits.

z = 2052           Same but for Teletype

z = 6144           Punch character from lowest 8 bits

z = 6148           Same but for Teletype

z = 7168           Program terminate to lower interrupt

level It starts in level 1 and can

descend to level 4.

Other IO instructions exist to control

a second reader, punch and Teletype, to

read status information, and to run

magnetic tape and plotters.

END OF THIS DOCUMENT

--------------------