Decoding DVZ the VZ300 DOS Floppy Disk Image Format

As used by DSVZ200 the VZ200 and VZ300 emulator

The VZ300 Double Density 5.25 Floppy Disk

Each disk has a 78K capacity, a total of 640 sectors per disk arranged as 40 tracks of 16 sectors per track in an interleaved arrangement. The image file size is 98,560 bytes. Track 0 (Sector 0 - 14) contains 112 directory entries; sector 15 of track 0 is a map of used sectors or write-protected sectors, leaving 624 free sectors (78K) for programs.

Image File Layout with interleaved sectors

|  | Track0 | Track1 | Track2 | Track3 | … |
| --- | --- | --- | --- | --- | --- |
| Sector0 | $9Abytes | $9Abytes | $9Abytes | $9Abytes | … |
| Sector11 | $9Abytes | $9Abytes | $9Abytes | $9Abytes | … |
| Sector6 | $9Abytes | $9Abytes | $9Abytes | … |  |
| Sector1 | $9Abytes | $9Abytes | … |  |  |
| ... | … | … |  |  |  |

Tracks

There are a total of 40 tracks per disk, each track is 2464 bytes with the image starting at track 0 sector 0, tracks are arranged 0 to 39 with 16 interleaved Sectors per Track.

Sectors

There are a total of 640 sectors per disk, sectors are 154 bytes arranged as in this Sector Order table.

Sector Order Table

| 0 | 11 | 6 | 1 | 12 | 7 | 2 | 13 | 8 | 3 | 14 | 9 | 4 | 15 | 10 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Sectors consist of four sections; a newly formatted image has only headers written on the image.

Sector Format

| OFFSET | BYTES | SECTION |
| --- | --- | --- |
| 0 | 24 | HEADER |
| 24 | 126 | DATA BYTES |
| 150 | 2 | NEXT TRACK AND SECTOR |
| 152 | 2 | 16 BIT ADD CHECKSUM |

Sectors begin with the sector header of 24 bytes, in each sector header there are three variables TR, SC and T+S. TR is the track, SC is the sector and T+S is track + sector used as a checksum.

Sector Header Format

| GAP1 | 80 | 80 | 80 | 80 | 80 | 80 | 00 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IDAM | FE | E7 | 18 | C3 | TR | SC | T+S |  |  |  |
| GAP2 | 80 | 80 | 80 | 80 | 80 | 00 | C3 | 18 | E7 | FE |

Sector Checksum Algorithm

The data section of each sector has a checksum found at the last two bytes of each sector. The checksum is a simple 16bit add equation of the data section (excluding the sector header).

Sectors use the 16 bit add checksum method

(Checksum ++Byte) && $FFFF

Sector Map

Track 0 sector 15 is the sector map, only the first 78 bytes are used the DOS ROM does not in any way use the remaining 50 bytes.

The map sector should be found located at an offset 2310 bytes from the start of the image file, two bytes are used per track, one bit per sector; 0 empty and 1 used. The sector map begins at the first byte of the data section; bytes 24 to 102.

Note that the sector map does not include track 0 and the sector maps bits are not arranged out of order.

Map word

Low byte					High byte

| B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 | B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 | S9 | SA | SB | SC | SD | SE | SF |

Directory

Track 0 sector 0 to 14 holds the directory information for the disk. There are 112 entry places; 16 bytes per entry, 8 entries per sector.

Entry elements and their offsets

|  | filetype | delimiter | filename | track | sector | startaddr | filesize |
| --- | --- | --- | --- | --- | --- | --- | --- |
| bytes | 1 | 1 | 8 | 1 | 1 | 2 | 2 |
| offset | 0 | 1 | 2 | 10 | 11 | 12 | 13 |