TI-85 Emulator Version 2.01�

����������������������������

This program can be found at:

ftp://users.aol.com/davidells/TI-Calc/

Make sure you download both the program and the ROM. They are

in separate ZIP files to save time when downloading new versions,

since the ROM must only be downloaded once.

Credits

�������

Z80 core based on Amstrad Emulator by

Bernd Schmidt (crux@pool.informatik.rwth-aachen.de)

TI-85 emulation by (sorted by last name):

David Ellsworth (davidells@aol.com)

Ilya Haykinson (haykinso@ucla.edu)

Dines Justesen (c958362@student.dtu.dk)

Robert Taylor (rtaylor@rtaylor.u-net.com)

History

�������

Version 1.00�

Version 1.50�

The following modifications were made by David Ellsworth:

* Accurate TI-85 interrupt handling

* Keyboard interrupt handler, allowing multiple

keys to be read at once by the TI-85

* Timer interrupt handler at true speed (200 Hz)

* ON key interrupts

* Linking port emulation through parallel cable

* Ability to save and load images of the TI-85's state

Version 2.00�

The following modifications were made by David Ellsworth:

* Replaced the Z80 core with one written in assembly. The

emulator is now *dramatically* faster. In fact, it runs

faster than a real TI-85 on my 486DX-25.

* Added support for the gray Graphlink cable and the

Connect-85 serial cable. The link emulation can now

be switched from the command line.

(Note: the Graphlink support does not quite fully work yet.)

* Added command line option to change the backup file name

Keyboard layout

���������������

Note: Many TI-85 keys have alphabetic and symbolic equivalents. I

have not listed those here.

TI-85 key               PC key

���������               ������

ON                      \

2nd                     Left Shift

ALPHA                   Left Ctrl

EXIT                    Esc

MORE                    Tab

x-VAR                   '

DEL                     Pad Insert or Pad Delete

GRAPH                   F7

STAT                    F8

PRGM                    F9

CUSTOM                  F10

CLEAR                   Backspace

Cursor keys             Pad Cursor keys

F1 - F5                 F1 - F5

A - Z                   A - Z

0 - 9                   0 - 9 or Num 0 - 9

+                       Pad+

-                       Pad-

*                       Pad*

/                       / or Pad/

^                       `

(-)                     - or Space

STO>                    =

(                       [

)                       ]

,                       ,

.                       . or ;

Special keys

������������

F11                     Quit emulator and save state

F12                     Quit emulator without saving state

Startup instructions

��������������������

Upon startup, this emulator will try to load a backup file. If you

did not specify one at the command line, it will be TI85.IMG. If it

cannot find this file, it memory will be initialized. You'll get the

message "Mem cleared/Defaults set", and a checkerboard cursor. Press

the [ON] key (backslash).

Command line parameters

�����������������������

L:Px

Use a parallel cable on LPTx.

If "x" is omitted, it will default to LPT1.

L:Sx

Use a (black) serial cable on COMx (not implemented yet)

If "x" is omitted, it will default to COM1.

L:Gx

Use a gray TI-Graph Link cable on COMx (not implemented yet)

If "x" is omitted, it will default to COM1.

L:K

Use PC speaker (as if it were plugged into the link port)

B:file

Load the backup image "file". If you quit the emulator and

save a backup, this file will be updated instead of TI85.IMG.

Backup files (state images)

���������������������������

When you use F11 to quit the emulator, its state will be saved as

TI85.IMG. When you start it again, this file will be loaded. You can

override the default name of TI85.IMG using a command line parameter.

Note: The backup file format was changed in version 2.00�.

The current file format is as follows:

Offset          Length        Description

������          ������        �����������

0x0000          0x8000        Image of RAM

0x8000          0x0002        AF register

0x8002          0x0002        BC register

0x8004          0x0002        DE register

0x8006          0x0002        HL register

0x8008          0x0002        PC register

0x800A          0x0002        AF' register

0x800C          0x0002        BC' register

0x800E          0x0002        DE' register

0x8010          0x0002        HL' register

0x8012          0x0002        R register (F)

0x8014          0x0002        R register (Fhi)

0x8016          0x0002        I register

0x8018          0x0002        imode (IM0, IM 1, IM 2)

0x801A          0x0002        IX register

0x801C          0x0002        IY register

0x801E          0x0001        IFF1 register

0x801F          0x0001        IFF2 register

0x8020          0x0002        SP register

0x8022          0x0002        Video offset (((port[0] & 0x3F) + 0xC0) << 8)

0x8024          0x0001        Current ROM page (port[5])

0x8025          0x0001        Keypad Mask (port[1])

0x8026          0x0001        Interrupt Mask (port[3])

0x8027          0x0001        Interrupt Flag (port[3])

0x8028          0x0001        Power Register (port[6])

0x8029          0x0001        Contrast Value (port[2])

0x802A          0x0001        Link Register (port[7])

Linking port information

������������������������

This program can emulate a linking port through a parallel cable.

Here is the plan for the cable. If you are cutting a TI link cable,

use this plan. If you are connecting the cable you make to one end

of a TI link cable (you don't want to cut one), swap ring and tip.

����������������������������������������������������Ŀ

� Parallel        TI-Link cable                      �

� (DB25)          (2.5mm 3cond phone)                �

�                                                    �

�   2  �ĳ#Ŀ                      �Ŀ               �

�   12 ���������  red (ring)       � � tip           �

�   3  �ĳ#Ŀ                      �Ĵ               �

�   13 ���������  white (tip)      � � ring          �

�                                  �Ĵ               �

�   22 ���������  shield (sheath)  � � sheath        �

�                                  ���               �

�                                                    �

�The diodes can be any small silicon switching diode.�

������������������������������������������������������

To tell the emulator what kind of cable you are using, pass the

appropriate command-line parameters.

The ability to send and receive 85G and 85B files will be

implemented in a future version.