$$l$0aCFN$0aR$00-1W1PIBOX DOCUMENTATION-0W0

The PIBOX may be used with or without  the  Disk  Controller.  If

used  with  the  DC then  LK1 should be removed. If a 1K resistor

has been inserted in place of LK1  then  the  end  connecting  to

IC419  pin  5  should be shorted to 5 volts. A switch may be used

for this purpose.

With the link inserted (or the switch  open  circuit)  the  Pibox

EPROM  slot  is  enabled.  The  EPROM may be either 8 or 16K. The

unit powers up in 8K mode and to enable  the  other  half  paging

must  take  place.  The  EPROM  occupies  the 8/9 slot or 6/7/8/9

depending on the paging register. An 8K  EPROM  will  wrap  round

from  8/9  to 6/7 if 16K paging is invoked. Similarly the 8K CMOS

RAM occupies the 8/9 slot and wraps round  in  16K  mode.  (Under

CP/M  with  the EPROM mode disabled the EPROM or RAM can be paged

into the 6/7 slot).

-1PORT ADDRESSES-0

192 - 195     PIO   (DA,DB,CA,CB)

196 - 199     CTC

200 - 203     ACIA

204 - 207     STATUS  (PAGING)

-1STATUS REGISTER-0

D7     D6         D4         D1     D0

.........................................

16/8   SP        V24/P       P1     P0

.........................................

0      X           X          0      0    =   8K EPROM (8/9)

1      X           X          0      0    =  16K EPROM (6/7/8/9)

0      X           X          1      1    =   8K RAM  (8/9)

1      X           X          1      1    =   8K RAM  (8/9 +6/7)

X = don't care

These are  the  only  permissable  combinations  for  the  memory

paging.  If the DC is enabled (LK1 inserted) then the PIBOX EPROM

is disabled in the 8/9 slot. The  6/7  slot  is  still  paged  as

above.  Whenever the EPROM or CMOS RAM is paged into the 6/7 slot

the Video RAM is of course paged out. It is not paged out however

from  the  video  circuitry  and  will  still  display correctly.

X      X           0          X      X    =  serial printer  tx

X      X           1          X      X    =  serial comms    tx

(receive data is always from comms)

SP is a spare connection on the PCB and may be used  for  control

purposes  by  those  building  extra  circuitry  onto  the board.

-1PIO-0

port   192        PIO port A    Data

port   193        PIO port B    Data

port   194        PIO port A    Control

port   195        PIO port B    Control

To access port A in bit control mode.

OPEN#194,7,194:OPEN#192,7,192

PUT#194,207,0 :REM set all bits to o/p

PUT#192,255   :REM set all lines high

PUT#192,0     :REM set all lines low

For a full description of the PIO it is recommended that the user

buys a Z80 PIO data book.

-1CTC-0

port 196          CTC channel 0     receive  baud rate

port 197          CTC channel 1     transmit baud rate

port 198          CTC channel 2     prescaler for channels 0 +1

port 197          CTC channel 1     transmit baud rate

OPEN#196,7,196:OPEN#197,7,197:OPEN#198,7,198

PUT#196,2

PUT#197,2

PUT#198,1

This sets the transmit and receive baud rates to 9600.

-1ACIA-0

The ACIA chip is the 6850.

port 200          ACIA Status

port 201          ACIA Data

The 6850 is difficult to use and  a  description  is  beyond  the

scope of this document. Users should buy a 6850 data book if they

wish to fully understand this device.

OPEN#200,7,200

PUT#200,3     :REM this resets the ACIA chip

PUT#200,17    :REM Sets 8 data 2 stop no parity

PUT#201,1     :REM Transmits 1 regardless of any handshake.

GET#201,a:?a  :Reads the input buffer and prints. The status

register should be checked for validity.

$$$0c-1CONNECTIONS-0

The connector functions available from the front are as follows.

CENTRONICS      V24      PRINTER      NB EXP           POWER

26 way      10 way     10 way      50 way           6 way

The connections at the rear of the board are as follows.

USER PORT       GFG PORT

20 way         26 way

All except the 6 way PSU connector are industry standard IDC

connectors.

The  V24 and serial printer are pinned out in such a way that a 6

way connector like the PSU one can be used in  place  of  the  10

way.  This will allow GBS serial cables to be used since they are

compatible with the NB serial ports.

V24    (serial comms RS 232)       PRINTER  (serial RS 232)

1      RX                           1       -

2      CTS                          2       CTS

3      -                            3       -

4      TX                           4       TX

5      RTS                          5       -

6      GND                          6       GND

7      DCD                          7       -

8      -                            8       -

9      +5 volts                     9       -

10      Spare pad                   10       -

CENTRONICS                           USER PORT

1         PB1   (Strobe)            1,3        +5 volts

3         PA0                       2          PA1

5         PA1                       4          PA0

7         PA2                       6          PB0

9         PA3                       8          PB1

11         PA4                      10          PB2

13         PA5                      12          PB3

15         PA6                      14          PB4

17         PA7                      16          PB5

21         PB 0  (Busy)             18          PB6

2 - 18       GND                     20          PB7

5 -19       GND

GFG PORT

1,2       GND                       7        Clock (4mhz)

3,4       + 5 volts                 5        spare

14        ARDY                      6        BRDY

16        ASTB                      15       BSTB

17        PA0                       8        PB0

18        PA1                       9        PB1

19        PA2                      10        PB2

20        PA3                      11        PB3

21        PA4                      12        PB4

22        PA5                      13        PB5

23        PA6                      24        PB6

25        PA7                      26        PB7

$$$0c-1Pibox Parts List-0

PCB (KD10016)

1      74HC00                  412

2      74HCO2                  414,421

1      74HC163                 420

2      74HC32                  409,418

1      74HC04                  407

1      74HC08                  419

1      74HC74                  413

2      74HC138                 415,417

1      74HC174                 416

1      74HC245                 402

1      1488                    406

1      1489                    408

DISCRETES

1      2N3904                  28

1      BST72A                  53

(Drain=C,Gate=B,Source=E)

7      1N914            11,13,17,38,45,8,41

T  T  L  B  L B  L (+ direction)

2      BZY88 4C3               24,6(+T)

1      10UF                    15

4      47UF                    7,14,9,39

14      0.1UF                   30,31,32,33,34,36,40,44,47,48,

49,50,55,56

4      0.22UF                  2,3,4,10

3      100R SIL                21,22,35

3      100R                    16,23,58

1      470R                    12

4      1K0                     37,42,27 +LK1,52,59,57

1      2K2                     51

1      3K9                     29

1      50W CONN

2      26W HEADER

2      10W    "

1      20W    "

Items marked #, are not needed unless an external PSU is to be

used. (Also the following components may be omitted - 8, 13, 14,

15, 17)

1#      6W CONN

1#      7805                     5     (mount on heatsink)

1#      79L12                    1

Items marked *, are not needed if an external PSU is to be used.

1*      7660                     401

1      40SKT

3      28W SKT

1      24W SKT

3      TC  links   18, link2,PBO(21)

1      AL PLATE   optional

Screws pillars etc

1      MOD CASE

1      Z80A-CTC                 410

1      Z80A-PIO                 411

1      ACIA  (6850)             405

50W-50W CABLE ASSY

1      BATTERY                  19

1      8K CMOS RAM (6264LP-15)  403

If  HC  gates are used or if only some of the LS and MSI are used

it may  not  be  necessary  to  have  an  external  PSU  and  the

associated circuitry.

NB:  If an external PSU is used the track from pin49 of the 50way

connector should  be  cut  between  the  2  holes  close  to  the

connector. This will allow a link to be inserted across the break

if the PSU is subsequently not used.

-1LINKS:-0

The centronics link should be made to 21

LK1 is inserted if an EPROM  is  present.(Will  not  work

with  DC)  LK1 may be a 1K resistor. A switch can then be

added to short IC419-5 to 5volts.This  will  inhibit  the

EPROM and a DC can be used with the unit.

ITEM  18 is shown as a cap. Use a TC link to short across

it. Spare DIL Pads. These are included to enable users to

build their own circuitry onto the board.

The  10  way  serial  connectors  are  intended  to  take

industry standard ribbon cable connectors.

$$$0cW1-1PI BOX Serial SoftwareW0-0

The PI BOX Serial Software consists of a set of additional device

drivers enabling the use  of  the  RS232  COMMs,  RS232  printer,

Centronics  type  port  and extra 8K CMOS battery backed RAM in a

similar manner to devices 0-11. Where ever possible the

parameters  and  usage of the PI BOX devices appear to be similar

to those found on the Grundy Business Systems Expansion Interface

Module  and  the  GFG Microsystems Micropage with DATAPACK and 8K

CMOS RAM.

Since  the  PI  Box  operates  with  or without a disk controller

software is supplied  in  EPROM  as  well  as  on  cassette.  The

cassette  software  is  intended  to  be  transferred to diskette

(avoiding the problem of supplying  on  different  disk  formats,

particularly  when  the  purchaser may not yet have disk drives).

When connecting a disk controller, the EPROM must be switched out

(a  switch  is  supplied fitted on built units for this purpose).

When  used  without  a Disk Controller TOP (of RAM) is lowered to

31744. This space is used for the RS232 buffer. The  buffer  size

is fixed (256 bytes for each buffer) and cannot be altered by the

user.

If  a disk system is being used with the PI Box then the software

is loaded at 29696.

Additional software is available on diskette (state format),  see

price  list.  This software allows CP/M to use the Centronic type

port  for  listings.  The  RS232  can  also  be  installed  (this

installation  however,  at present, has no interrupt handler, and

may loose characters if proper handshake is  not implemented with

the external device [computer, printer]). The Snap Camera

software and PIO Calibration software are also provided.

The PI Box Serial software is a full and proper extension to  the

NewBrain  input output system (IOS), and is available to any user

program or machine code routines.

The device table is extended to 21.

-1DEV10           Device 10-0

the dummy device is replaced by a new device that

re-configures your system to 40K BASIC. The extra

devices are then lost, and  the  normal  NewBrain

IOS restored.

PORT            ignored

PARAMETER       none

Example         OPEN#10,10

-1ACIALST         Device 16-0

RS232  printer.  It's  use is to the exclusion of

the RS232 COMMS device 17. Behaves as  device  8,

but  has  the  'L'  option  to define line length

(before carriage return line feed).

PORT            Only one ACIA device may be opened at a time.

PARAMETER       see ACIADEV.

INPUT           error 109: Input from printer.

OUTPUT          see ACIADEV.

-1ACIADEV         Device 17-0

RS232 COMMS. It's use is to the exclusion of  the

RS232 printer device 16. Behaves as device 9 with

several extensions  as  device  17  on  the  EIM.

PORT            Only one ACIA device may be opened at a time.

PARAMETERS      In any order and case blind.

'D'     Data bits               {7,8}

default 8

'I'     immediate return (particularly for

terminal use)

'L'     line length             {0..255}

default 0

'P'     Parity odd or even      {O,E,Z}

default no parity

'R'     receive baud rate       {75..19200}

default 9600

'S'     Stop bits               {1,2}

default 1 (*)

'T'     transmit baud rate      {75..19200}

default 9600

* Note: Device 9 has 2 stop bits, fixed.

Permissable combinations:

8 data, 2 stop bits, and no parity set

8 data, 1 stop bits, and no parity set (default)

8 data, 1 stop bits, and odd, even or no parity

7 data, 2 stop bits, and odd or even parity

baud rates :

75, 150, 300, 600, 1200, 2400, 4800, 9600, 19200.

split baud rates, range:

75...9600, 150..19200, ie. not 75 and 19200

Examples        OPEN#17,17,"iT1200R75":REM immediate return on no

data, transmitting at 1200 baud, receiving at 75

baud, no parity, 8 data, 1 stop bit .

OPEN#16,17,"T9600L120":REM printer set at 9600

baud with line length 120, using COMMS port.

OPEN#16,16,"T1200D7PES2":REM printer port,

1200 baud, 7 data even parity and 2 stop bits.

Errors:         210:    Buffer empty or no data to input

215:    Illegal combination of DATA bits, STOP

bits and parity requested.

216:    Framing error (as EIM ACIA)

217:    overrun or BUFFER overrun (as EIM ACIA)

-1TERMDEV         Device 18-0

Mini-Terminal device. Requires  stream  1  to  be

opened as  a screen of 80 columns and 25 rows and

stream 17 to be opened in immediate mode  and  at

the appropiate baud rate (with device 17).

If  the  stop  key is pressed during power up the

Mini-terminal is immedately entered and streams 1

and 17 opened.

PORT            Ignored

PARAMETER       none. The Copyright notice and date of version

are returned in FILE$ to BASIC or length in reg

BC and start of string in HL after the call to

open, ie PRINT FILE$.

INPUT           none

OUTPUT          0 characters are reduced to 7-bit (top bit

zeroed).

1 returns to 8-bit data mode (default).

Notes:          The terminal uses ZKLOOK to scan the keyboard.

ADM-3A type terminal screen codes

CTL-C   clear to end of line

CTL-Z   clear screen

ESC *   clear screen

ESC =   set cursor absolute, row, column, offset

32 e.g. the four btyes ESC = y+32 x+32

ESC T   clear to end of line

ESC t   clear to end of line

Normal NewBrain screen codes  are  not  affected.

In  particular,  highlight on is CTL-N, highlight

off is CTL-O.  Set  Terminal  with  CTL-W  D  and

re-set with CTL-W B.

Device 19 not used.

-1SMDDEV          Device 20-0

8K  Serial  memory.  The  optional  8K RAM can be

battery backed. That is, when the  power  is  off

memory  is  retained. Up to 3 months is possible,

but 1 month retention is more normal. The battery

trickle  charges  when the NewBrain is on and the

PI Box connected. This device is identical to the

DATAPACK  device  13.  Data  and  programs can be

saved to the RAM and later read  back  into  main

RAM.

PORT            Even ports => 8K with 8/9 slot CMOS RAM. Odd port

=> 16K EPROM. Multiple copies of device, allowing

for pointers at different locations.

PARAMETER       start address. Defaults to 8000H for RAM or 8800H

for EPROM.

Examples        OPEN#20,20,"38000". Points to RAM at address

38000.

OPEN#20,20,1,"40000". Points to EPROM at address

40000.

INPUT           Reads location and increments pointer.

OUTPUT          Writes to location if <A000H then increments

pointer. Error flagged if >=A000H.

Error           131 end of RAM: as in end of tape

Note:  If  used  in  RAM  (with  disk  ROM  being

paged into the  8/9  slot),  drives  loaded  from

disk/tape  then  the  8K CMOS RAM is paged in the

6/7 slot and the 8K ROM paged in the 6/7 slot has

starting address default 6800H.

-1CENTDEV         Device 21-0

Centronics type device.

PORT            Opening with even port (default) causes line feed

to be appended to carriage return.  May  also  be

used  to  make  multiple  copies  of  the device.

PARAMETER       Line length, default 80.

Examples        OPEN#21,21,1,"L0":REM output true, ie. no line

feed after carriage return, no line length.

OPEN#21,21,"L132":PUT#21,15:REM set condense mode

on printer (EPSON code) and append LF to CR on

the 132nd character.

INPUT           Returns 1 = busy, 0 = not busy.

OUTPUT          Tabs are expanded. The  multiple  copies  of  the

device enables switching between text and

graphics  on  the  printer  (after  sending   the

appropiate  code  to the printer when switching).

WARNING:  Do  not  connect  electronic devices to

the other Z80 PIO ports when using  the  printer.

The  cables  can be left connected, but output to

the printer may echo to  the  User  and  parallel

ports.  Similarly  output, say to the sound board

may produce effects on the printer.

240386 GFG

$$$0c                      PIBOX COSTINGS

1)      PCB                     10.48

2)      50W CONN                 1.00

3)      26W HEADER               1.50

4)      10W    "                 1.00

5)      26W    "                 1.50

7)      6W CONN                  1.00

8)      7805                     0.40

9)      7660                     1.50

10)     40SKT                    0.48

11)     28W SKT  *3              0.75

13)     74HC00                   0.30

14)     74HCO2                   0.30

15)     1488                     0.30

16)     1489                     0.30

17)     74LS163                  0.75

18)     74LS32                   0.20

19)     74LS04                   0.13

20)     74LS08                   0.16

21)     74LS138                  0.30

22)     74LS32                   0.20

23)     74LS174                  0.40

24)     74LS138                  0.30

25)     74LS02                   0.15

26)     74LS74                   0.19

27)     74LS245                  0.46

28)     DISCRETES                1.00

29)     AL PLATE                 1.50

----------

TOTAL             26.55

MOD CASE                 3.00

CTC                      2.50

PIO                      2.50

ACIA                     2.50

20 WAY USER              1.00

6  WAY POWER CONN        1.00

50W-50W CABLE ASSY       2.00

BUILD COST     10.00

---------

51.05

PROFIT      15.32

--------

66.37

VAT         09.96

--------

76.33           76.33           76.33

6)      BATTERY                  2.00

12)     8K CMOS RAM              3.50

SOFTWARE  DEV DRIVERS                            25.00          25.00

TERMINAL  OR PRESTEL                                  35.00

------            ------         ------

81.83           101.33         136.33