$$l$0aCFN$0a(R$00

W1-1DATAPACK SERIALW0-0

W1-1DEVICE DRIVER SOFTWAREW0-0

The  DATAPACK serial device driver software converts the NewBrain

micropage board into a high  speed  'silicon  tape'  giving  both

serial and random access to bytes, strings and programs.

The  DATAPACK  software overwrites device 10 at powerup, provides

new devices 12 and 13  and,  in  addition  the  usual  facilities

available  from  a  NewBrain  device  driver,  may be used as the

console (including from powerup) providing the NewBrain with full

turnkey operation. A program to enable the user to implement this

quickly is also available.

The data held in any mix of 8 or 16K EPROMS may  be  accessed  by

all  of  the usual means (GET PUT LOAD MERGE INPUT LINPUT PRINT).

Since the data is held in a read only medium, clearly one  cannot

SAVE  a  program to the device driver associated with the EPROMs.

Output sent to the device is interpreted as  stream  of  commands

and command arguments. These commands greatly enhance the

device's usefulness providing access to over 60K  of  data  area.

For  example,  one  can easily select the 37th string, or the 4th

program or the second program in EPROM number 3 without having to

know  where the data is physically stored. Detection of 8 and 16K

EPROMS is performed by the device driver as is all  of  the  slot

manipulation.  The  only errors reported by the device driver are

'bad open as not enough memory' (it does  needs  a  tiny  bit  of

workspace!),  'bad  parameters on open' and 'no more data' (error

code 133 -  just  like  the  cassette  driver  for  convenience).

-1SUMMARY OF DEVICE DRIVER SPECIFICATION-0

DEVICE NUMBER:  10

OPEN IN/OUT:

PORT:           Specifies  the  EPROM Slot (defaults to 0) to be

started with. In the case of  slot  0  (default),

the  pointer  is  set to that byte directly after

the device driver code, otherwise it is the  byte

at location 8000H.

PARAMETERS:     The first two bytes (default to 0) represent the

absolute (CAREFUL!) start address of the  pointer

(MSB,LSB)  and  assumes  an 8K EPROM - useful for

loading RAM with test data.

e.g.

OPEN#10,10,2,"33500" opens stream 10 on device 10

with port 2 and sets  the  read  pointer  to  the

absolute address of 33500 in slot 2 (ie the third

EPROM from the left). Therefore you can open up 4

streams,  each pointing to a different slot and a

different start address.

OPEN#10,10           :REM slot 0, starting at 32678

OPEN#101,10,1        :REM slot 1, starting at 32678

OPEN#102,10,2,"33312":REM slot 2, starting at 33312

OPEN#103,10,3,"40000":REM slot 3, starting at 40000

NOTE:           slot  3  is  assumed  by  device  13  to be RAM.

Device 10 assumes it to be EPROM. As the  devices

are  independant  you can write data using device

13 and read it using devices 10 or 13.

On OPEN#10,10,n,"nnnn" printing file$ returns the

Copyright notice and issue number.

FILE$ (only if the parameter string is

entered, eg. OPEN#10,10,"32768") returns to BASIC

the Copyright notice and date of issue.

INPUT:          The byte being pointed to is read. If  it  is  an

acceptable byte then it is returned otherwise the

pointer is ADVANCEd. If no error occurs then  the

code  is  re-executed otherwise error code 133 is

returned.

ADVANCED NOTE:  8K  EPROM  are  straight  forward.  The  pointer

advances,  starting  at  8000H, to 9FFFH and then

onto the next EPROM at 8000H. However 16K  EPROMs

behave  as two 8K EPROMs and depending on how you

start to read them.

eg.1 Slot 1 contains 8K EPROM, slot 2 16K EPROM.

RESERVE TOP-6144 : OPEN#10,10,1,"36480" :

PUT#10,5:REM read 255s

FOR i=0 TO 163*128: GET#10,a:POKE TOP+i,a:NEXTi

:CLOSE#10

Will  read  bytes  from the 3712th byte in the 8K

EPROM (at address 36480 up to 40959 (end  of  the

8K EPROM), then start reading from the first byte

in the 16K EPROM  at  address  24576  up  to  the

16384th  byte  at address 40959. The file (HiSoft

Pascal  1.5A  infact)  is  transferred  to   RAM.

eg.2 Slot 3 contains 16K EPROM.

RESERVE TOP-10000 : OPEN#10,10,3,"40448" :

PUT#10,5:REM read 255s

FOR i=0 TO 68*128 :GET#10,a: POKE TOP+i,a

:NEXT i:CLOSE#10

Will read bytes from address 40448  up  to  40959

then  read from address 24576, ending at 32767 in

the  same  EPROM.  Again,  the  file  (this  time

GEN31.BIN,  of  DEVPAC 3), is transferred to RAM.

OUTPUT:         The sent byte is decoded as being a command or  a

command argument depending on its context. If  in

doubt, sending a sequence of six 0's (nulls) will

reset  the  context  to  'expecting  a  command'.

Internally, there are six bytes of  storage  that

may  be set by the user. These are grouped into 3

16 bit words and called Par0 Par1 and Par2.  Some

commands assume that these parameters have

already been set, some do not.

The command bytes are as follows:

0) Null         No action

1)  Set Par0 LSB only to the next byte

2)  Set Par0 MSB and then LSB to following 2

bytes

3)  Ignore all further OUTPUT (unless re-opened

of course)

4)  Ignore all 255 characters (default)

5)  Read all characters

6)  Specify Par2, Par1 and Par0 values with the

next six bytes interpreted as Par2 MSB,LSB,

Par1 MSB,LSB, Par0 MSB,LSB

7)  Set pointer to the start of (Par0)th string

from the start of the data in the current

EPROM slot. This ammounts to 'Rewind' if Par0

is zeroed.

8)  Skip the next (Par0) strings.

9)  Skip the current file (ie look for 13,4,13 as

eof).

10) Set pointer to the byte following the next

occurance of the byte in Par0 LSB.

11) Set pointer to read from 1st byte in slot

Par0 LSB.

12) Set pointer to absolute address in Par0 - use

with CARE!!

13) Null        No action

14) Buzz with Par2 units high, Par1 units low for

Par0 cycles. NOTE: Buzz may be aborted using

the STOP key with no effect on the rest of

the program i.e. does not produce a BREAK

condition for BASIC.

NOTES:

a)  LSB/MSB stand for Last/Most Significant Byte.

b)  A 'string' is any sequence of chars ending in

CHR$(13).

c)  A 'file' is any sequence of bytes terminated

by a string followed by a string containing

only a CHR$(4) i.e. the byte sequence

13,4,13.

d)  PUT#10,1,m,n,12 will set the pointer to

256*m+n. If this value is below 32678 (that

is in RAM) then unexpected results may occur.

Pointer Advance Algorithm.

The pointer is incremented.

If the pointer is at  A000H  then  the  EPROM  is

examined  for  being  an  8 or 16K EPROM (this is

done by assuming it to be 16K and  comparing  the

top  half  with the bottom half until either they

differ [and is therefore a 16K EPROM]  or  8K  is

identical  and  so is taken to be 8K). If it is a

16K device then the pointer is set to  6000H.  If

it is 8K then the pointer is set to 8000H and the

EPROM slot incremented - if this is more  than  3

then the error code 133 is returned.

If the pointer is at 8000H (a 16K EPROM must have

just been fully read)  then  the  EPROM  slot  is

incremented  and, if it is more than 3, the error

code 133 is returned.

DEVICE NUMBER:  12

OPEN IN/OUT:    PORT: For multiple copies of the device.

PARAMETERS: Number of 256 byte blocks, similar to

TAPE1 and TAPE2.

Example : OPEN#12,12,"*6"

INPUT:          The byte being pointed to is read. If  it  is  an

acceptable byte then it is returned otherwise the

pointer is ADVANCEd. If no error occurs then  the

code  is  re-executed otherwise error code 133 is

returned.

OUTPUT:         Commands passed to device driver.

0)  rewind input pointer to beginning

1)  delete character at input pointer - store as

next character for input unless another

output

2)  skip past occurance of next character

3)  dump out output,input,max

ie return the 6 bytes for the write pointer,

read pointer and maximum size of the buffer,

offset from start of device own memory.

4)  null

5)  append from input

6)  set input absolute to following 2 bytes.

ie set read pointer to number of bytes from

the beginning of the devices' own memory.

7)  set output absolute to following 2 bytes.

ie set write pointer to number of bytes from

the beginning of the devices' own memory.

8)  backspace input

17) set insert mode (until another command sent,

or input requested).

27) Escape next character

needed to store bytes that otherwise would be

commands eg PUT#12,27,4 to store ASCII

control D.

DEVICE NUMBER:  13

OPEN IN/OUT:    PORT: For multiple copies of the device.

INPUT:          The byte being pointed to is read. The pointer is

ADVANCEd.

OUTPUT:         The byte is written at the position  pointed  to.

If  this  is  RAM  then  the pointer is advanced.

Attempts to write to ROM are  checked  and  error

193 returned.

-1SUMMARY OF THE DATAPACK MENU PROGRAM-0

The program is designed to be located  just  after  the  DATAPACK

software in the first EPROM on the board (slot 0).  If  the  stop

key  is  pressed  during  powerup  the program is loaded and run.

Alternatively you can enter

OPEN#0,10

The  program scans the EPROMs for program names until no more can

be found. This fact is noted by  either  an  error  133  (end  of

'tape') or a 'program title' of "END". The accumulated titles are

displayed and the user invited to either select one or return  to

BASIC.  If  a program is selected then the program deletes itself

up to line 64999, MERGEs the program source  code  and  RUNs  the

program.  The  program  must  NOT use line 65535. If the autoboot

version is being used and if the program terminates  by  'droping

through  towards  line  65000 then the program will close all the

streams, reset the ON ERROR and  ON  BREAK  conditions  and  then

OPEN#0,10.  This  has the effect of reloading the menu program as

in powerup and producing the menu.

When scanning for a program, the software expects  to  find  each

program preceeded by a string containing the name of that program

from which  the  menu  entries  are  constructed.  All  CHR$(255)

characters  are  ignored  when scanning for a program. This means

that one may choose to pack the programs  very  tightly  to  save

space  (not  an easy thing to do at first) or start them from the

beginning of an EPROM - recommended until you  get  used  to  the

idea,  and  has the added advantage of making programs physically

interchangable.

This means that a program looks like ...

'name of program',13,13,'line',13,'line',13 ... 'line',13,4,13

... where 'name of program' is a valid string (not in quotes) and

'line' is a valid source line (tokenised by SAVE or expanded by

LIST).

NOTE: The 'end of program' is ONLY registered by a 13,4,13

sequence. Saving the program with SAVF makes loading much faster

(disk users are given this extra BASIC command).

Any 255 bytes are ignored whether they are inside or outside the

program. This means that spare space in EPROMS should be left as

255 bytes.

As  the  sequence  13,4,13  defines  the  end of file and hence a

possible file name as the next string, any string read is assumed

to be less than 40 bytes. Some machine routines have the 'end-of-

file' sequence in the middle of the code  (  valid  machine  code

instructions),  but  they  are  then  followed  by  a 'string' of

considerable length. The menu program ignores these 'false'  eofs

and counts the number of dummies. After the selection the pointer

is rewound to the beginning (PUT2,0,0,7:REM console is device 10,

after  OPEN#0,10).  Then  the  number  of  eofs  found  up to the

selection are skipped. The filename is read, and if this is still

a  match to the title found by the menu on the screen the program

is loaded. Stream 0 is reopened on device 0 (in case of  errors).

Therefore,  if the BASIC program looks for a file or machine code

it must look for the file or pre-presumes that the file is  in  a

fixed location.

eg  The  Pascal  loader  assumes  that the machine code starts at

36480 in slot  0.  The  Devpac  loader,  however  looks  for  the

Micropage  entry name, hence finding the machine code in any slot

1 to 3 (it can not be placed in slot 0, since if it were then the

unit would always display a Micropage type header).

Normal BASIC programs can be placed end to end.

eg.

'TITLE 1',13,13,first prog 1st line,13,....nth program

line,'TITLE 2,13,13,second prog 1st line,...,etc.

Programs must start with a 'null string' ie just a NEWLINE,

(which is supplied by BASIC when the program is SAVEd or SAVFed).

Programs can be loaded by OPEN#10,10,p:REM p for start of pth

EPROM. Followed by as many PUT#10,9 as required to to skip

unwanted files, and finally a LOAD#10 {or MERGE#10:RUN}.

There are two additional short routines available at the  end  of

the DATAPACK code.

1) DEVREStore

This  tests  the  E/F  to check for Series 2 software and

restores the orginal device table. This  is  required  by

software that wishes to page in the 8K CMOS RAM and leave

it in the 8/9 slot, and hence  can  not  use  the  device

drivers in the DATAPACK EPROM.

CALL 34768: REM TOP+2000, fixed, ie the JumP table entry

$$$0c2) RAMTESt

This is a quick test for RAM in the 4th slot (ie slot 3).

If  RAM  is  not  present  ERROR  91  (not  verified)  is

returned.

The machine code performs the equivalent of :-

r=FALSE : CLOSE#255 : OPEN#255,13,255 : GET#255,a

: PUT#255,a+1+254*(a>255) : GET#255,b : CLOSE#255

: IF a<>b THEN r=TRUE : OPEN#255,13,255

: PUT#255,a: CLOSE#255

This allows programs to quickly test for the availablity

of the 8K CMOS RAM by

CALL 34770: REM TOP+2002, ie the JumP table entry

If error = 91 then there is no RAM.

NOTE    If a program requires to use the 8K CMOS in slot 3 then

it must page this slot in. eg

10 ONERRORGOTO20

:CALL 34770          {error 91 means no RAM}

:?"40K"              {RAM found}

:POKE7,160           {Update B3PRM for RAM TOP}

:CALL 34768          {disable new devices}

:OPEN#7,7,128:PUT#7,128+9:CLOSE#7 {page in slot 3}

:GOTO40

20 RESUME 30            { error, ie no RAM}

30 ?"32K"               { only normal 32K system}

...

If you have machine code to load then add the sequence

DATA 62,137,211,128,201

then, instead of 'OPEN#7,7,128:PUT#7,128+9:CLOSE#7' CALL

ad where ad is the address where the 5 bytes were loaded.

This is         LD A,128+9

OUT (128),A

RET

*****************************************************************

**** JUST IN CASE ANYONE IS THINKING A LITTLE POORLY, NO YOU ****

**** CANNOT USE NORMAL MICROPAGE PROGRAMS WITH THE DATAPACK  ****

**** SOFTWARE ...                                            ****

*****************************************************************

Authors Postscript.

The  DATAPACK  device  driver  software,  DATAPACK  menu program,

micropage supervisor program, XNDISC, XCONFIGUR, Terminal

emulator  program,  RELOC,  DEREM  SHRINK  (NewBrain BASIC source

compression program), PAGES, MPRELOC,  XBIOS,  PIBOX  Serial  and

several  other  programs  are  all Copyright (c) N.C.Bane. If you

frequently rip me off by copying the software without payment  of

a  license  fee  then  I won't find it worth my while to sell any

more.