MB-02+

HARDWARE DOCUMENTATION

© 1995-2002 8BC

RTC:

LOW ADDRESS 3, %00000011, #03; TYPE: OUT, IN

| HIGH ADDRESS | A3 | A2 | A1 | A0 | REGISTER | BIT3 | BIT2 | BIT1 | BIT0 | VALUE | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0, #00 | 0 | 0 | 0 | 0 | S1 | s4 | s3 | s2 | s1 | 0 - 9 | 1 - second digit register |
| 1, #01 | 0 | 0 | 0 | 1 | S10 | * | s30 | s20 | s10 | 0 - 5 | 10 - second digit register |
| 2, #02 | 0 | 0 | 1 | 0 | MI1 | mi4 | mi3 | mi2 | mi1 | 0 - 9 | 1 - minute digit register |
| 3, #03 | 0 | 0 | 1 | 1 | MI10 | * | mi30 | mi20 | mi10 | 0 - 5 | 10 - minute digit register |
| 4, #04 | 0 | 1 | 0 | 0 | H1 | h4 | h3 | h2 | h1 | 0 - 9 | 1 - hour digit register |
| 5, #05 | 0 | 1 | 0 | 1 | H10 | * | AM/PM | h20 | h10 | 0 - 2 | AM/PM, 10 - hour register |
| 6, #06 | 0 | 1 | 1 | 0 | D1 | d4 | d3 | d2 | d1 | 0 - 9 | 1 - day digit register |
| 7, #07 | 0 | 1 | 1 | 1 | D10 | * | * | d20 | d10 | 0 - 3 | 10 - day digit register |
| 8, #08 | 1 | 0 | 0 | 0 | MO1 | mo4 | mo3 | mo2 | mo1 | 0 - 9 | 1 - month digit register |
| 9, #09 | 1 | 0 | 0 | 1 | MO10 | * | * | * | mo10 | 0 - 1 | 10 - month digit register |
| 10, #0A | 1 | 0 | 1 | 0 | Y1 | y4 | y3 | y2 | y1 | 0 - 9 | 1 - year digit register |
| 11, #0B | 1 | 0 | 1 | 1 | Y10 | y40 | y30 | y20 | y10 | 0 - 9 | 10 - year digit register |
| 12, #0C | 1 | 1 | 0 | 0 | W | * | w3 | w2 | w1 | 0 - 6 | Week register |
| 13, #0D | 1 | 1 | 0 | 1 | RegD | 30sec ADJ | IRQ FLAG | BUSY | HOLD | - | Control register D |
| 14, #0E | 1 | 1 | 1 | 0 | RegE | t1 | t0 | ITRPT /STND | MASK | - | Control register E |
| 15, #0F | 1 | 1 | 1 | 1 | RegF | TEST | 24/12 | STOP | RESET | - | Control register F |

Bit * does not exist.

Please mask AM/PM bit with 10's of hours operations.

Busy is read only, IRQ can be set low only.

| Data Bit | AM/PM | ITRPT/STND | 24/12 |
| --- | --- | --- | --- |
| 1 | PM | ITRPT | 24 |
| 0 | AM | STND | 12 |

ITRPT/STND = INTERRUPT/STANDARD

HARDDISK:

ADDRESS 7, %00000111, #07; TYPE: OUT, IN

DMA:

ADDRESS 11, %00001011, #0B; TYPE: OUT, IN

FDC:

ADDRESS 15, %00001111, #0F; TYPE: OUT, IN

ADDRESS 47, %00101111, #2F; TYPE: OUT, IN

ADDRESS 79, %01001111, #4F; TYPE: OUT, IN

ADDRESS 111, %01101111, #6F; TYPE: OUT, IN

FLOPPY:

ADDRESS 19, %00010011, #13; TYPE: OUT

BIT0		/ACTIVE A

BIT1		/MOTOR ACTIVE A

BIT2		/ACTIVE B

BIT3		/MOTOR ACTIVE B

BIT4		/ACTIVE C

BIT5		/MOTOR ACTIVE C

BIT6		/ACTIVE D

BIT7		/MOTOR ACTIVE D

ADDRESS 19, %00010011, #13; TYPE: IN

BIT0		/DRQ

BIT1		/DISK CHANGE

BIT2		/INTRQ

BIT3		HDIN

BIT4		/MOTOR ACTIVE A

BIT5		/MOTOR ACTIVE B

BIT6		/MOTOR ACTIVE C

BIT7		/MOTOR ACTIVE D

MEMORY:

ADDRESS 23, %00010111, #17; TYPE: OUT

BIT0 - BIT4	page (0-31)

BIT5		WRITE ENABLE

BIT6		SRAM ENABLE

BIT7		EPROM ENABLE

If BIT6 and BIT7 are set /RESET is generated.

BACK-UP BATTERY INFORMATION:

Lithium-based battery used for RTC & memory back-up.

Type: CR-2032

PIO:

BASE ADDRESS 31, %0xx11111, #1F; TYPE OUT, IN

or

BASE ADDRESS 27, %0xx11011, #1B; TYPE OUT, IN

| xx - A6, A5 | REGISTER |
| --- | --- |
| 00 | PA |
| 01 | PB |
| 10 | PC |
| 11 | CWR |

CANNON 37:

| PIN | FUNCTION |
| --- | --- |
| 1 | GND - ground |
| 2 | PA0 |
| 3 | PA1 |
| 4 | PA2 |
| 5 | PA3 |
| 6 | PA4 |
| 7 | PA5 |
| 8 | PA6 |
| 9 | PA7 |
| 10 | PB0 |
| 11 | PB1 |
| 12 | PB2 |
| 13 | PB3 |
| 14 | PB4 |
| 15 | PB5 |
| 16 | PB6 |
| 17 | PB7 |
| 19 | GND - ground |
| 20 | UCC - power 5V |
| 21 | PC0 |
| 22 | PC1 |
| 23 | PC2 |
| 24 | PC3 |
| 25 | PC4 |
| 26 | PC5 |
| 27 | PC6 |
| 28 | PC7 |
| 36 | */RESET - output only! |
| 37 | UCC - power 5V |

CANNON 9:

| PIN | FUNCTION | KEMPSTON | MOUSE |
| --- | --- | --- | --- |
| 1 | PA3 | UP | HOR. 1 |
| 2 | PA2 | DOWN | VER. 1 |
| 3 | PA1 | LEFT | HOR. 2 |
| 4 | PA0 | RIGTH | VER. 2 |
| 6 | PA4 | FIRE | LEFT BUTTON |
| 7 | UCC - power | UCC - power | UCC - power |
| 8 | GND - ground | GND - ground | GND - ground |
| 9 | PA5 | - | RIGHT BUTTON |

PRINTER:

| CANNON 25 | CANNON 37 | FUNCTION | TYPE |
| --- | --- | --- | --- |
| 1 | PC1 | /STROBE | output |
| 2-9 | PB0-PB7 | D1-D8 | output |
| 10 | PC2 | /ACK | input |
| 11 | PC7 | BUSY | input |
| 12 | PC6 | PAPEROUT | input |
| 13 | PC5 | SELECT | input |
| 14 | PC3 | /AUTOFEED | output |
| 15 | PC4 | /ERROR | input |
| 25 | GND | GND | ground |

8BC Laboratories Index: 9708122112-8BCL