ZX-Evolution

Configuration by TS-Labs

User Guide

Features

• High compatibility with original Pentagon clone

• Advanced video features:

– Raster sizes: 360x288, 320x240, 320x200, 256x192 of standard pixel resolution

– Up to 720x288 Hi-res pixel resolution

– Hardware scrolled graphic planes

– 256 and 16 indexed colors per pixel

– Programmable palette up to 15625 colors using PWM video-DAC modulation

– Handy 512 and 256 bytes per line addressing

– Text mode with loadable font and hardware line-fine scroll

– Up to 256 displayable video pages

• Hardware engine for Tiles and Sprites graphics

– Up to 42 sprites per line (and non-limited sprites per frame using Sprite Reuse)

– Sprites sized up to 64x64 pixels individually

– Up to 3 prioritized sprite planes

– Up to 2 prioritized tile planes with 8x8 pixels tiles

– Tiles and sprites have ‘transparent’ color

– Tiles and sprites have same graphics format allowing unified graphics processing

– Individually programmable horizontal per-line pixel-fine scrolls for tiles planes

– Individually programmable vertical per-column line-fine scrolls for tiles planes

– Up to 16 palettes for sprites per line

– Up to 4 palettes for tiles per line for each tile plane

• Z80 Memory addressing enhancements

– Programmable RAM page for any 16kB window individually

– ‘Shadow’ RAM pages for #8000 and #C000 windows

• Z80 acceleration features

– 14, 7 and 3,5MHz speed modes

– 512 bytes of fast non-wait RAM for speed critical procedures for 14MHz speed

– On-the-fly programmable INT position

– Programmable IM2 vector

• Advanced hardware features

– DRAM-to-Device, Device-to-DRAM and DRAM-to-DRAM DMA Controller

1. Block diagram

2. Ports

2.1 Introduction

All extended ports are addressed via address pool #nnAF, where #nn is the HEX number of register. (All hexadecimal values are marked with # sign.)

(*Please refer to the to_do.xls document at the page #nnAF for short ports reference.)

3. Graphics

3.1 Introduction

The pixel data generated by the video rendering hardware has unified format and is transferred by the hardware using 8 bit data bus. These 8 bits of video data are split on two 4 bit pieces in hi-res modes to represent 2 adjacent pixels of higher resolution. The video data itself is not yet a color, but an index for palette, the input address for CRAM, the palette RAM. CRAM output is the real color displayed on TV or VGA display. See the chapter 0.0 for CRAM description. One lo-res pixel has a length of period of 7MHz clock, and one hi-res – 14MHz.

3.2 Raster

The video hardware generates all signals necessary for correct displaying the video on TV or VGA. Video signal has blanking fields and visible area. Each frame consists of 320 lines, and each line consists of 448 7MHz pixel periods. See the tables 3-1 and 3-2 for raster timings.

Table 3-1.   Vertical timings

| VCOUNTER value | Field |
| --- | --- |
| 0-31 | Vertical Blank |
| 32-319 | Vertical Visible Area |

Table 3-2.   Horizontal timings

| HCOUNTER value | Field |
| --- | --- |
| 0-87 | Horizontal Blank |
| 88-447 | Horizontal Visible Area |

3.3 Video Modes

The currently displayed graphics mode is programmed via register #00 (VConfig) of extended port #AF. It configures the pixel area resolution and the format of graphics. See the Table 3-3 for VConfig description.

| Register #nnAF | Name | Description |
| --- | --- | --- |
| #00 | VConfig | Video Configuration Register |

Table 3-3.   Video Configuration Register

| Bit(s) | Name | Value on Reset | Description |
| --- | --- | --- | --- |
| 7:6 | RRES | 00 | Raster Resolution (see Table 3-2) |
| 5:3 | - | 000 | Reserved |
| 2:0 | VM | 000 | Video Mode (see Table 3-3) |

Warning: any bits marked as ‘reserved’ should be written with zeros for future compatibility!

The visible area covers 360x288 pixels of standard (lo-res) ZX resolution. It consists of pixel area and solid color border. The RRES specifies the size of the pixel area. When RRES=11 the whole visible area is used by pixels, no border displayed.

Table 3-4.   Raster Resolution

| RRES | Resolution, pixels lo-res | Resolution, pixels hi-res |
| --- | --- | --- |
| 00 | 256x192 | 512x192 |
| 01 | 320x200 | 640x192 |
| 10 | 320x240 | 640x240 |
| 11 | 360x288 | 720x288 |

Table 3-5.   Video Modes

| VM | Name | Description |
| --- | --- | --- |
| 000 | ZX | 6144 (32x192) bytes of graphics, 768 (32x24) bytes of attributes |
| 001 | 16c | 131072 (256x512) bytes, 2 pixels per byte, 16 colors per pixel |
| 010 | 256c | 262144 (512x512) bytes, 1 pixel per byte, 256 colors per pixel |
| 011 | Text | 8192 (128x64) bytes of chars, 8192 (128x64) bytes of attributes |
| 100 | - | Reserved |
| 101 | - | Reserved |
| 110 | - | Reserved |
| 111 | - | Reserved |

16c, 256c and Text modes use the addressing suitable for effective programming. The size of graphics array in memory exceeds the visible screen and is never displayed at once. The visible window size is specified by RRES and its position is programmed by X and Y scroll registers (see chapter). Thus, moving the visible window across the graphics data can be achieved the effect of scrolling, without need of actual moving the data in memory.

4. Tile and Sprite Engine

5. Memory map

6. Z80 clock

7. Interrupts

8. DMA